SDK Overview

PrinterSDK_K388 primarily integrates UrovoSDK and printing-related code implementations to support scanning and printing functionalities.

SDK Function API Description

Scanning

Usage Steps

- Register broadcast listener in the page.

- Initialize scan configuration for broadcast output:

scanManager = new ScanManager(); scanManager.OpenScanner(); // Power on scanManager.SwitchOutputMode(0); // 0-Broadcast mode, 1-Textbox output

- Trigger scanning via code or hardware button:

scanManager.StartDecode();

- When finished, execute stopDecode(), closeScanner(), and unregister the broadcast receiver.

ScanManager API

| Type | Name | Description |
| --- | --- | --- |
| bool | closeScanner() | Turns off the barcode scanner power |
| void | enableAllSymbologies(boolean enable) | Enables/disables all supported barcode types |
| void | enableSymbology(Symbology barcodeType, boolean enable) | Enables/disables specified barcode type |
| int | getOutputMode() | Gets current scan result output mode |
| int[] | getParameterInts(int[] idBuffer) | Gets integer values of specified scanner configuration items |
| string[] | getParameterString(int[] idBuffer) | Gets string values of specified scanner configuration items |
| bool | getScannerState() | Gets scanner power state (On/Off) |
| bool | getTriggerLockState() | Gets scan trigger key lock state |
| Triggering | getTriggerMode() | Gets current trigger mode |
| bool | isSymbologyEnabled(Symbology barcodeType) | Checks if specified barcode type is enabled |
| bool | isSymbologySupported(Symbology barcodeType) | Checks if device supports decoding specified barcode type |
| bool | lockTrigger() | Locks scan trigger key (disables scan button) |
| bool | openScanner() | Powers on the barcode scanner |
| bool | resetScannerParameters() | Resets all barcode type settings to factory defaults |
| int | setParameterInts(int[] idBuffer, int[] valueBuffer) | Sets integer values for specified scanner configuration items |
| bool | setParameterString(int[] idBuffer, String[] valueBuffer) | Sets string values for specified scanner configuration items |
| void | setTriggerMode(Triggering mode) | Sets trigger mode controlling decode operations |
| bool | startDecode() | Starts barcode decoding |
| bool | stopDecode() | Stops ongoing data capture |
| bool | switchOutputMode(int mode) | Sets barcode scanner output mode (to textbox or Android Intent) |
| bool | unlockTrigger() | Unlocks scan trigger key (enables scan button) |

Printing

Basic Usage

- Initialize printer settings (e.g., paper width/height):

PrinterSDK printer = PrinterSDK.getInstance(); int maxWidth = 50 * 8; // Assuming 8 pixels per unit printer.pageSetup(maxWidth, 30 * 8);

- Add print content:

- // Text printer.drawText(0, currentY, "Label Print Test", fontType, fontSize, 0, 0, false, false); // Barcode printer.drawBarCode(0, currentY, "20230815143001", 1, 0, 3, 100); // Image printer.drawGraphic(0, currentY, bitmap);

- Start printing

printer.print(0, printListener); // 0: No paper feed, 1: Label detection, 2: Left black mark detection, 3: Right black mark detection

Advanced Usage

- Use PrinterConnector for printer connection (execute in sub-thread):

- PrinterConnector printerConnector = new PrinterConnector(); boolean open = printerConnector.open(); if (open) { printerConnector.flush(); PrinterConnector.PrinterResult status = printerConnector.status(); // ...

- Send custom print data (e.g., CPCL or other printer-supported commands):

- String cpcl = "! 0 200 200 240 1\r\n" + "CENTER\r\n" + "PAGE-WIDTH 400\r\n" + "T 24 1 0 0 Label CPCL Print Test\r\n" + "T 24 1 0 35 Barcode: 20230815143001\r\n" + "BARCODE 128 2 1 100 0 70 20230815143001\r\n" + "GAP-SENSE\r\n" + "FORM\r\n" + "PRINT\r\n"; byte[] bytes = cpcl.getBytes("GBK"); // Handle Chinese encoding byte[] gzipBytes = GZIPFrame.codec(bytes); // Send data boolean write = printerConnector.write(gzipBytes, gzipBytes.length); // Close connection printerConnector.close();

PrinterSDK API

| Type | Name | Description |
| --- | --- | --- |
| void | about() | Ignored command |
| void | addCommand(byte[] bytes) | Adds print command (extended use) |
| void | alignCenter() | Center alignment |
| void | alignLeft() | Left alignment |
| void | alignRight() | Right alignment |
| void | backGround(int level) | Watermark grayscale level |
| void | barcodeText(int font, int size, int offset, int rotate, int width, int ratio, int height, int x, int y, String data) | Barcode identifier |
| void | bkText(int font, int size, int x, int y, int lev, String str, int rotate) | Print watermark |
| void | contRast(int level) | Set contrast level{level}: 0=Default, 1=Medium, 2=Dark, 3=Very DarkHigher contrast makes prints darker but slower |
| void | count(int num) | Auto-increment/decrement number (±65535) |
| void | drawBarCode(int start_x, int start_y, String text, int type, int rotate, int linewidth, int height) | Print barcode |
| void | drawBox(int lineWidth, int top_left_x, int top_left_y, int bottom_right_x, int bottom_right_y) | Print rectangle box |
| void | drawGraphic(int start_x, int start_y, Bitmap bmp) | Print image |
| void | drawINVERSE(int x0, int y0, int x1, int y1, int width) | Print inverse line segment |
| void | drawLine(int lineWidth, int start_x, int start_y, int end_x, int end_y) | Print solid line |
| void | drawQrCode(int start_x, int start_y, String text, int rotate, int ver, int lel) | Print QR code |
| void | drawText(int text_x, int text_y, String text, int fontSize, int rotate, int bold, boolean reverse, boolean underline) | Print text (simplified) |
| void | drawText(int text_x, int text_y, String text, int fontType, int fontSize, int rotate, int bold, boolean reverse, boolean underline) | Print text |
| void | drawText(int text_x, int text_y, String text, String fontType, int fontSize, int rotate, int bold, boolean reverse, boolean underline) | Print text (font by name) |
| void | end() | End command |
| static PrinterSDK | getInstance() | Get PrinterManager instance |
| byte[] | getPrintBytes() | Get created print data for this session |
| void | multLine(int height, int fontType, int fontSize, int x, int y, int rotate, String... strs) | Multi-line printing |
| void | pageSetup(int pageWidth, int pageHeight) | Set page width/height |
| void | pageSetup(int pageWidth, int pageHeight, int rotation) | Set page width/height with rotation* |
| void | postfeed(int len) | Feed paper after printing |
| void | prefeed(int len) | Feed paper before printing |
| void | print(int skip, PrintListener printListener) | Start printingskip: 0=No feed, 1=Label detect, 2=Left black mark, 3=Right black mark |
| void | print(PrintListener printListener) | Start printing |
| void | printWait(int time) | Delayed printing (1/8 time unit) |
| void | setBold(int level) | Set bold font{level}: 0=Cancel bold, >1=Enable bold |
| void | setMag(int w, int h) | Character magnification |
| void | setPace() | Batch printing (press feed button for next label) |
| void | setPrintTime(int time) | Set print copies |
| void | setSP(int spacing) | Character spacing (spacing*0.125mm) |
| void | speed(int level) | Print speed{level}: 0-5 (0=Slowest) |
| void | textConcatenation(int x, int y, List<CONCAT> concat, int rotate) | Text concatenation |
| void | underLine(boolean mode) | Underline |
| String | version() | Version information |

PrinterConnector API

| Type | Name | Description |
| --- | --- | --- |
| void | close() | Disconnect printer |
| void | flush() | Clear print response buffer |
| boolean | isConnected() | Printer connection status |
| boolean | open() | Connect to printer |
| int | read(byte[] data, int readLen, int timeout_ms) | Read printer response |
| PrinterConnector.PrinterResult | status() | Get printer status |
| boolean | write(byte[] data, int len) | Send print data |
| boolean | write(String str) | Send print data (string) |

SDK Proguard Rules

Add the following rules when enabling ProGuard:

-keep class android.content.** { ; } . -keep class android.os.* { ; } . -keep class android.device.* { ; } . -keep class com.urovo.* { *; }